S33SDM

Suzuki GSR 750 2011-

Mounting kit:

| No. | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Slider holder | 537/B | 2 |
| 2 | Bolt M10x105x1,25 | 893 | 2 |
| 3 | Dural. cylinder d=37mm left side | 195 | 1 |
| 4 | Dural. cylinder d=35mm right side | 189 | 1 |
| 5 | Cone.spacer d=15mm | 9 | 2 |
| 6 | Bolt M6x20 | 1024 | 4 |
| 7 | Washer M6 | 1023 | 4 |

Installation:

- Dismantle original bolts from chassi.

- Insert ID.1, ID.5 and ID.3 on the bolt ID.2. (left side). This kit fix into the engine thread.

- Insert ID.1, ID.5 and ID.4 on the bolt ID.2. (right side). This kit fix into the engine thread.

- Place slider and use bolts ID.6 with washers ID.7 and mount slider on the aluminium holder.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

1

1

5

3

2

5

2

4

7

7

6

6