S33S

Suzuki GSR 750 2011-

| Kit pro levou stranu |  |  |  |  | Kit pro pravou stranu |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no | pcs |  |
| 1 | slider RDmoto | SL | 1 |  |  | 1 | slider RDmoto | SL | 1 |  |
| 2 | cover | SLcap.. | 1 |  |  | 2 | cover | SLcap.. | 1 |  |
| 3 | Moto adapter | DV-328 | 1 |  |  | 3 | Moto adapter | DV-320 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Bolt M10x75x1,25mm | 938 | 1 |  |  | 6 | Bolt M10x75x1,25mm | 938 | 1 |  |
| 7 | Washer M10 – spring | 801 | 1 |  |  | 7 | Washer M10 – flat | 801 | 1 |  |

Instructions (left side):

- Remove original bolt from frame.

- Insert bolt M10 into adapter and mount it into the frame of motorcycle. Spring washer M10 between frame of motorcycle and adapter.

- Mount slider on the slider adapter using three bolts M6.

- Mount cover into the slider using two bolts M4.

We recommend to use the glue on all threads (medium strong).

Instructions (left side):

- Remove original bolt from frame.

- Insert bolt M10 into adapter and mount it into the frame of motorcycle. Place flat washer M10 under bolt.

- Mount slider on the slider adapter using three bolts M6.

- Mount cover into the slider using two bolts M4.

We recommend to use the glue on all threads (medium strong).