S34SDM

Suzuki GSX-R1000 2017-

| LEFT SIDE KIT |  |  |  |  | RIGHT SIDE KIT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  | ID | Description | Cat.no. | Pcs |
| 1 | Dural cylinder 30mm | DV11 | 1 |  | 1 | Dural cylinder 30mm | DV11 | 1 |
| 2 | Bolt M10x60x1,25mm | 896 | 1 |  | 2 | Bolt M10x60x1,25mm | 896 | 1 |
| 3 | Dural cylinder 6mm | 1229 | 1 |  | 3 | Dural. cylinder 8mm | 1134 | 1 |
| 4 | Bolt M10x65mm | 943 | 1 |  | 4 | Bolt M10x65mm | 943 | 1 |
| 5 | Steel adapter S34-L | 717 | 1 |  | 5 | Steel adapter S34-P | 718 | 1 |
| 6 | Flat washer M10 | 804 | 1 |  | 6 | Flat washer M10 | 804 | 1 |
| 7 | Safety nut M10 | 810 | 1 |  | 7 | Safety nut M10 | 810 | 1 |
| 8 | Washer M10 (spring) | 801 | 2 |  | 8 | Washer M10 (spring) | 801 | 2 |
| 9 | SLDM adapter | 537/B | 1 |  | 9 | SLDM adapter | 537/B | 1 |
| 10 | Flat washer M6 | 1023 | 2 |  | 10 | Flat washer M6 | 1023 | 2 |
| 11 | Bolt M6x20mm | 1024 | 2 |  | 11 | Bolt M6x20mm | 1024 | 2 |
| 12 | Tape 3M | - | 1 |  | 12 | Tape 3M | - | 1 |

INSTRUCTIONS

- We recommend to use safety glue on all threads (medium strong).

- Dismantle side fairing of motorcycle.

- Unbolt original bolt M10 from the frame of motorcycle.

- Insert dural. Cylinder ID3 between steel adapter ID5 and frame of motorcycle. Fit steel adapter using botls ID2 and washer ID8.

- Mount side fairing back on motorcycle.

- Fit SDM adapters ID1 and ID9 using bolt ID4 and ID8.

- Mount safety ID7 nut on bolt ID4.

- Fit protector using two bolts ID11 and washers ID10.