S34S

Suzuki GSX-R1000 2017-

| LEFT SIDE KIT | RIGHT SIDE KIT |
| --- | --- |

| ID | Description | Cat.no. | pcs |  | ID | Description | Cat.no. | pcs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Slider adapter | 317 | 1 |  | 1 | Slider adapter | 317 | 1 |
| 2 | Bolt M10x40mm | 854 | 1 |  | 2 | Bolt M10x40mm | 854 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |  | 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |  | 4 | Bolt M4x16 | 876 | 2 |
| 5 | Bolt M10x60x1,25mm | 896 | 1 |  | 5 | Bolt M10x60x1,25mm | 896 | 1 |
| 6 | Alu. cylinder 6mm | 1229 | 1 |  | 6 | Alu. cylinder 8mm | 1134 | 1 |
| 7 | Steel adapter S34-L | 717 | 1 |  | 7 | Steel adapter S34-P | 718 | 1 |
| 8 | Washer M10 | 804 | 1 |  | 8 | Washer M10 | 804 | 1 |
| 9 | Safety nut M10 | 810 | 1 |  | 9 | Safety nut M10 | 810 | 1 |
| 10 | Spring washer M10 | 801 | 1 |  | 10 | Spring washer M10 | 801 | 1 |
| 11 | Tape 3M | - | 1 |  | 11 | Tape 3M | - | 1 |

INSTRUCTIONS

- Unbolt original side fairing and original bolt from the frame of motorcycle.

- Place washer ID10 on bolt ID5. Insert this kit through the adapter ID7 and alu cylinder ID6. Mount this kit into the frame of motorcycle.

- Place side fairing back on the motorcycle.

- Fit adapter ID1 using bolt ID2. Tighten it using washer ID8 and nut ID9.

- Fit slider using three M6 bolts.

- Fit cover using two M4 bolts.

We recommend to use safety glue on all threads (medium strong).