Instructions for the right side:

- Remove fairing of motorcycle.

- Remove original bolts from the frame of motorcycle.

- Mount dural. adapter ID1 into the frame of motorcycle using bolts ID4.

- Put dural. connection rod ID6 through the frame and use bolt ID5.

Instructions for the left side:

- Remove fairing of motorcycle.

- Fit dural adapter ID1 on the frame of motorcycle. Tighten it usin stirrup ID2 and bolt ID6.

- Stick 3M tape on the frame of motorcycle.

- Tighten rod using bolt ID5

Now put fairing back on motorcycle.

Instructions for crash protectors:

- Insert bolt ID4 through ID9, ID7 and ID3.

- Fit this kit into the adapter ID1.

- Fit protectors using bolts and washers (ID10 and ID11).

We recommend to use safety glue on all threads (medium strong).

S35SDM

Suzuki  GSX 1250 FA-2010

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 664 | 1 |
| 2 | Threaded stirrup | 665 | 1 |
| 3 | Dural. cylinder 90mm | 143 | 1 |
| 4 | Bolt M12x135x1,25 | 991 | 1 |
| 5 | Bolt M6x30 | 863 | 1 |
| 6 | Bolt M6x50 | 867 | 2 |
| 7 | Dural. cylinder 15mm | 149 | 1 |
| 8 | 3M Tape | 3M | 1 |
| 9 | Dural. adapter for SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 666 | 1 |
| 2 | Dural. cylinder 90mm | 143 | 1 |
| 3 | Bolt M12x135x1,25 | 991 | 1 |
| 4 | Bolt M10x50x1,25 | 921 | 2 |
| 5 | Bolt M6x30 | 863 | 1 |
| 6 | Dural. rod | 667 | 1 |
| 7 | Dural. cylinder 15mm | 149 | 1 |
| 8 | 3M tape | 3M | 2 |
| 9 | Dural. adapter for SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |

Kit for the left side				Kit for the right side

3

4

9

8

9

6

3

2

1

2

5

1

7

6

7

5

4