S35S

Suzuki GSX 1250FA 2010

Kit for left side:					Kit for right side:

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 1 | Dural.holder for SL01 | 664 | 1 |
| 2 | Dural. special clevis | 665 | 1 |
| 3 | Dural.cylinder | 296 | 1 |
| 4 | Dural adapter | DV314 | 1 |
| 5 | Bolt M12x110x1,25 | 910 | 1 |
| 6 | Bolt M6x30 | 863 | 1 |
| 7 | Bolt M6x50 | 867 | 2 |
| 8 | Slider | SL01 | 1 |
| 9 | Cap | SLcap | 1 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolt M4x16 | 876 | 2 |
| 12 | 3M tape | 3M | 2 |

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 1 | Dural.holder for SL01 | 666 | 1 |
| 2 | Dural.cylinder | 296 | 1 |
| 3 | Dural. adapter | DV314 | 1 |
| 4 | Bolt M12x110x1,25 | 910 | 1 |
| 5 | Bolt M10x50x1,25 | 921 | 2 |
| 6 | Bolt M6x30 | 863 | 1 |
| 7 | Dural.bar | 667 | 1 |
| 8 | Slider | SL01 | 1 |
| 9 | Cap | SLcap | 1 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolst M4x16 | 876 | 2 |
| 12 | 3M tape | 3M | 2 |

8

9

8

10

11

10

11

5

2

6

7

1

4

3

1

3

5

4

6

7

12

2

Instruction right side:

- Take out fairing.

- To give way bolts on coolant.

- Dismantle originál bolts from frame and mount dural. bracket ID.1 use bolts ID5.

- Mount duralumin bar ID.7 and use bolts ID.6.

Instruction left side:

- Take out fairing.

- Mount dural. bracket ID.1 use special clevis from back side ID.2 and use bolts ID7.

- Stick 3M tape on the frame ID.12, not to scratch frame

- Mount dural. bar use bolt ID.6

Mount fairing and coolant back to bike.

Instruction to assembly for protectors:

- Insert bolts ID. 5 into the dural.adapter for SL01 ID.4 a ID.3 after dural.cylinder ID.3 and ID.2

- This set mount on the bike.

- Mount slider ID.10, insert colour caps ID.9, use bolts ID.11

Use glue of all threads.