Instruction right side:

- Take out fairing.

- To give awaz bolts from coolant.

- Take out originál bolts from frame and mount dural. bracket ID.1 use bolts ID4.

- To put through dural.bar ID.6 use bolts ID.5

Instruction left side::

- Take out fairing.

- Mount dural.bracket on the frame ID.1 use special clevis ID.2 and bolts ID.6.

- Mount dural. bar. And use bolt ID.5

Mount fairing coolant back to bike.

Instruction to assembly crash protectors:

- Insert into the protektor steel box ID.9

- To put through bolts ID. 4 a ID. 3 M12x135x1,25 to crash protektor.

- Put on the bolt dura. cylinder ID.2 a ID. 3

- This set mount on the bike

Use glue of all threads.

S35

Suzuki  GSX 1250 FA-2010

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 1 | Dural.bracket | 664 | 1 |
| 2 | Dural. special clevis | 665 | 1 |
| 3 | Dural.cylinder d=90 | 143 | 1 |
| 4 | Bolt M12x135x1,25 | 991 | 1 |
| 5 | Bolt M6x30 | 863 | 1 |
| 6 | Bolt M6x50 | 867 | 2 |
| 7 | Crash protektor | PH | 1 |
| 8 | 3M tape |  | 1 |
| 9 | Steel box | X2 | 1 |

| ID | Description | Cat.n | Qty |
| --- | --- | --- | --- |
| 1 | Dural.bracket | 666 | 1 |
| 2 | Dural.cylinder d=90 | 143 | 1 |
| 3 | Bolt M12x135x1,25 | 991 | 1 |
| 4 | Bolt M10x50x1,25 | 921 | 2 |
| 5 | Bolt M6x30 | 863 | 1 |
| 6 | Dural.bar | 667 | 1 |
| 7 | Crash protector | PH | 1 |
| 8 | 3M tape | 3M | 2 |
| 9 | Steel box | X2 | 1 |

Fitting kit left side:				Fitting kit right side: