S39

Suzuki DL 1000 V-Strom 2014-

| Fitting kit left side: Fitting kit right side: ID. | Descrpition | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Dural. spacer d=60mm | 1194 | 1 |
| 2 | Special nut | X12/X2 | 1 |
| 3 | Stainless rod 12x(1,25)x445x30x30 |  | 1 |

| ID. | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 1 | Dural. spacer d=70mm | 296 | 1 |
| 2 | Special nut | X12/X2 | 1 |

Instruction to assembly:

- Take out original bolt M12 from frame.

- Through the frame put stainless rod ID.3.

- From the left hand side insert dural. spacer ID.1 d=60mm.

- From the right hand side insert dural. spacer ID.1 d=70mm.

- Insert protectors on the stee rod.

- Tighten protectors by special nuts from both side.

Warning, use glue of threads, medium strong.