S3

Suzuki SV650 N/S 2003-2006

Fitting kit left side:		     		  Fitting kit right side:

| ID | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Screw M10x110x1,25 | 928 | 1 |
| 3 | Dural.cylinder d=20 | 182 | 1 |

| ID | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Screw M10x160x1,25 | 929 | 1 |
| 3 | Dural.cylinder | 181 | 1 |

2

1

1

2

3

3

Assembly make to the frame, position on photos. Make first left side.

Assembly to left side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector and after insert bolt ID.2.

- Take dural.cylinder ID.3 and insert it on the bolt.

- This kit mount into the frame.

- Thighten the safety bolt on frame.

Assembly to right side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector and after insert bolt ID.2.

- Take dural.cylinder ID.3 and insert it on the bolt.

- This kit mount into the frame.

Warning:

We recommend use the glue of all threads. Medium strong.