S41

Suzuki GSX-S1000F ´16-

Left side:		     				Right side:

| ID | Description | Cat. | Qt |
| --- | --- | --- | --- |
| 1 | Steel insert | X1 | 1 |
| 2 | Bolt M10x90x1,25 | 890 | 1 |
| 3 | Alu.spacer d=20 | 1188 | 1 |

| ID | Description | Cat. | Qt |
| --- | --- | --- | --- |
| 1 | Steel insert | X1 | 1 |
| 2 | Bolt M10x100x1,25 | 892 | 1 |
| 3 | Alu.spacer d=21,5 | 1189 | 1 |

1

3

2

2

3

1

Instructions:

- Start with the left side.

- Insert ID.1 into the protectors.

- Place alu spacers under protectors. For the left side use bolt M10x90 for tightening.

- For the right side use bolt M10x110 for tightening.

Note: Use gule of all threads during assembly.