S42

Suzuki SV 650 2018-

Mounting Kit Left Side:		     		Mounting Kit Right Side:

| ID | Description | Cat.No. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. roller d=52,5 | 1309 | 1 |
| 2 | Steel nut M12 | X12/X2 | 1 |
| 3 | Rod M12x430x1,25x30x30 |  | 1 |

| ID | Description | Cat. No. | Pcs. |
| --- | --- | --- | --- |
| 1 | Dural. roller d=52,5 | 1309 | 1 |
| 2 | Steel nut M12 | X12/X2 | 1 |

1

Mounting instructions for both sides:

- Unscrew original M12 bolt going through the motorcycle frame.

- Take the ID.3 M12 rod and push it through the frame. Then equip it with ID.1 roller, crash protector and M12 ID.2 steel nut from both sides.

- Finally, tighten all bolts on both sides.

Note: We recommend using medium strenght thread glue during assembly (blue color).