S4

Suzuki SV 1000

| Fitting kit left side: Fitting kit right side: ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. spacer d=33mm | 47 | 1 |
| 2 | Bolt M10x140x1,25 | 923 | 1 |
| 3 | Steel sleeve | X1 | 1 |

| Assembly: ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. spacer d=35mm | 46 | 1 |
| 2 | Bolt M10x120x1,25 | 895 | 1 |
| 3 | Steel sleeve | X1 | 1 |

- Dismantle original bolts from chassi.

- Insert steel sleeve ID.3. into the protectors. Under the protectors place alu. spacers. For left side spacer lenght 35mm. Tighten it by bolt ID.2 M10x140x1,25.

- Assembly for right side is same. There are only different snes bolt and spacer.

Note: We recommend to use medium glue for threads during assembly.

2

3

2

1

1

3