S5

Suzuki GSX-R600 SRAD 1997-2000

Suzuki GSX-R750 SRAD 1996-1999

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | QTY |  |  | ID | Description | Cat.No. | QTY |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x105x1,25mm | 893 | 1 |  |  | 2 | Bolt M10x105x1,25mm | 893 | 1 |  |
| 3 | dural. cylinder d=40mm | 45 | 1 |  |  | 3 | dural. cylinder d=30mm | 44 | 1 |  |

Instruction to Assemble:

- Remove the original bolts from frame.

- Put on the bolts steel box (ID1). Insert the dural. cylinder (ID3) under the bolt (ID2) and fixed, on the left side.

- Under the protector on the right side, insert dural cylinder d=30mm (ID3) and use bolt (ID2).

- The first time make left side and after do right side.

PH01

2

1

3

Warning - for the threads of the screws is recommended to use premium adhesive moderate (blue color)

2

3