S8S

Suzuki GSX 650 Bandit ´07-

Left side kit:		     		       	Right side kit:

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Bolt M8x40 | 828 | 2 |
| 2 | Alu.adapter | 568 | 1 |
| 3 | Alu. cylinder | 187 | 1 |
| 4 | Steel adapter | 566 | 1 |
| 5 | Bolt M8x20 | 823 | 2 |
| 6 | Bolt M12x40 | 858 | 1 |
| 7 | Bolt M8x30 | 834 | 1 |
| 8 | Washer M8 | 802 | 1 |
| 9 | Slider adapter | 350 | 1 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.no | Pcs |
| --- | --- | --- | --- |
| 1 | Alu. cylinder | 186 | 1 |
| 2 | Bolt M10x30 | 882 | 1 |
| 3 | Plastic cap | X4 | 1 |
| 4 | Steel stirrup | 950 | 1 |
| 5 | Nut M8 | 951 | 2 |
| 6 | Washer M8 | 802 | 2 |
| 7 | Bolt M12x40 | 858 | 1 |
| 8 | Steel adapter | 567 | 1 |
| 9 | Slider adapter | 350 | 1 |
| 10 | Bolt M6x25 | 877 | 3 |
| 11 | Bolt M4x16 | 876 | 2 |
| 12 | 3M tape | 3M | 1 |

ID4

2

ID3

ID8

ID7+8

ID2

ID7

ID5

ID1

ID4

ID9

ID9

ID5+6

ID2

ID6

ID5

1

ID1

ID10

ID11

ID12

ID3

ID10

ID11

Instructions – left side:

- Remove original bolts M8 from the frame of motorcycle.

- Mount adapter ID2 on the frame of motorcycle using bolt ID1.

- Mount alu. Cylinder ID3 using bolt ID7+ID8 into the steel adapter ID4.

- Mount adapter for slider ID9 on the steel adapter ID4 using bolt ID6.

- Fit this kit on the frame using two botls ID5.

- Fit slider using three bolts ID10.

- Fit coloured cover using two bolts ID11.

Instructions – right side:

- Remove nut from the frame of motorcycle. Mount cylinder ID1 into the thread.

- Mount slider adapter ID9 on the steel adapter ID4 using bolt ID7.

- Stick 3M tape on the frame of motorcycle.

- Place „U“ stirrup ID4 and tighten it using nuts and washers ID5 and ID6.

- Tighten bolt ID2 with alu. Cylinder ID1. Cover bolt with plastic cap ID3.

- Fit slider using three bolts ID10.

- Fit coloured cover using two bolts ID11.

We recommend to use the glue on all threads (medium strong).