T10S

Triumph Speed Triple 1050 2011-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no. | Pcs |  |  | ID. | Description | Cat.no. | Pcs |  |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |  |
| 3 | Adapter for SL01 | DV-318 | 1 |  |  | 3 | Adapter for SL01 | DV-318 | 1 |  |  |
| 4 | Botl M10x30mm | 882 | 1 |  |  | 4 | Bolt M10x30mm | 882 | 1 |  |  |
| 5 | Bolt M6x25 | 877 | 3 |  |  | 5 | Bolt M6x25 | 877 | 3 |  |  |
| 6 | Bolt M4x16 | 876 | 2 |  |  | 6 | Bolt M4x16 | 876 | 2 |  |  |
| 7 | Duralumin nut M22x1,5mm | 446 | 1 |  |  | 7 | Duralumin nut M22x1,5mm | 446 | 1 |  |  |
| 8 | Duralumin bolt M22x1,5mm | 447 | 1 |  |  | 8 | Duralumin bolt M22x1,5mm | 447 | 1 |  |  |
| 9 | Bolt M12x80mm | 956 | 1 |  |  | 9 | Bolt M12x80mm | 956 | 1 |  |  |
| 10 | Washer M12 | 803 | 1 |  |  | 10 | Pad M12-plate | 803 | 1 |  |  |
| 11 | Nut M12-safety | 811 | 1 |  |  | 11 | Nut M12-safety | 811 | 1 |  |  |
| 12 | Washer M10-spring | 801 | 1 |  |  | 12 | Washer M10-spring | 801 | 1 |  |  |

Instruction to assembly:

- remove the original bolt M12 from frame

- take the duralumin nut ID.7 insert the bolt M12 ID.9 into the nut

- mount dural.nut and bolt into the frame, use washer ID.10 and nut ID.11

- mount dural.bolt ID.8  into the dural.nut

- take the adapter for SL01 ID.3 insert into the adapter bolt M10x30 ID.4 and washer-spring ID.12

- mount slider ID.1, use bolts ID.5 and insert cover into the slider ID.2 use bolts ID.6

Mounting instruction for second side is same.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

9

7

8

3

4,12

1

10,11

5

6

2

1

3

4,12

8

7

10,11

9