T11S

Triumph 1200 Explorer

Kit for left side:				         Kit for right side:

| Note | Description | Cat.No | Quant |
| --- | --- | --- | --- |
| 1 | Dural.adapter d=60mm | 318 | 1 |
| 2 | Bolt M10x45 | 848 | 1 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Bolt M6x25 | 877 | 3 |
| 5 | Dural. cylinder- 13mm | 1100 | 1 |
| 6 | Bolt M12x70x1,25 | 937 | 1 |
| 7 | Dural.spacer- 50mm | 1101 | 1 |
| 8 | Bolt M12x120x1,25 | 913 | 1 |
| 9 | Washer M12 | 803 | 2 |
| 10 | Steel adapter for slider | 657 | 1 |

| Note | Description | Cat.No | Quant |
| --- | --- | --- | --- |
| 1 | Dural.adapter d=40mm | 317 | 1 |
| 2 | Bolt M10x45 | 848 | 1 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Dural. cylinder- 13mm | 1100 | 1 |
| 5 | Bolt M12x70x1,25 | 937 | 1 |
| 6 | Dural. cylinder- 43mm | 1102 | 1 |
| 7 | Bolt M12x120x1,25 | 913 | 1 |
| 8 | Washer M12 | 803 | 2 |
| 9 | Steel adapter for slider | 658 | 1 |
| 10 | Bolt M6x25 | 877 | 3 |

Mounting instruciton- left side:                         Mounting instruction- right side:

- Dismantle original bolts from frame.

- For front side used duraumin cylinder (note 6) and back side insert cylinder (note 4).

- For front side mounting use the bolt (note 7) and washer (note 8) used the original nut, for rear side use bolt (note 5) and washer (note 8).

- Insert bolt (note 2) into the dural.adapter (note 1) and mounting it on the steel adapter. Check photo for right position of slider.

- Slider is mounting bolts (note 10) and cover bolts (note 3).

- Dismantle original bolts from frame.

- For front side use dural. spacer (note 7) and into the back side cylinder (note 5)

- For front side mounting use the bolt (note 8) and washer (note 9) use the original nut. For mounting use bolt (note 6) and washer (note 9).

- Insert bolt (note 2) into the dural.adapter (note 1) and mounting it on the steel adapter. Check photo for right position of slider.

- Slider is mounting bolts (note 4) and cover bolts (note 3).

Warning- we recommended use the glue on all threads

9

10

7

8,9

4

3

10

6,9

1

3

7,8

1

6

5

4

5,8

2

2