T11

Triumph 1200 Explorer

Kit for left side:				         Kit for right side:

| Note | Description | Cat.No | Quant |
| --- | --- | --- | --- |
| 1 | Crash protectors | PH | 1 |
| 2 | Steel box | X1 | 1 |
| 3 | Dural. cylinder- 35mm | 159 | 1 |
| 4 | BoltM10x80mm | 852 | 1 |
| 5 | Dural. cylinder- 13mm | 1100 | 1 |
| 6 | Bolt M12x70x1,25 | 937 | 1 |
| 7 | Dural.cylinder- 50mm | 1101 | 1 |
| 8 | Bolt M12x120x1,25 | 913 | 1 |
| 9 | Washer M12 | 803 | 2 |
| 10 | Steel adapter for slider | 657 | 1 |

| Note | Description | Cat.No | Quant |
| --- | --- | --- | --- |
| 1 | Crash protector | PH | 1 |
| 2 | Steel box | X1 | 1 |
| 3 | Bolt M10x50mm | 849 | 1 |
| 4 | Dural. cylinder- 13mm | 1100 | 1 |
| 5 | Bolt M12x70x1,25 | 937 | 1 |
| 6 | Dural. cylinder- 43mm | 1102 | 1 |
| 7 | Bolt M12x120x1,25 | 913 | 1 |
| 8 | Washer M12 | 803 | 2 |
| 9 | Steel adapter for slider | 658 | 1 |

Mounting instruciton- left side:

Warning- we recommended use the glue on all threads.

- Insert the steel box into the crash protector. (note  2)

- Used bolt (note 4) and insert duralumin cylinder (note 3) under the bobbins.

- This set mounting on the adapter (note 10).

- Dismantle original bolts from frame.

- For front side use dural. cylinder (note 7) and into the back side cylinder (note 5)

- For front side mounting use the bolt (note 8) and washer (note 9) use the original nut. For mounting use bolt (note 6) and washer (note 9).

- Put on the bobbins colour cover, If you have PHV1 or PHV2.

Mounting instruction- right side:

Warning- we recommended use the glue on all threads

- Insert the steel box into the crash protector. (note  2)

- Use bolt (note 3) and adapter (note 9).

- Dismantle original bolts from frame.

- For front side used duraumin cylinder (note 6) and back side insert cylinder (note 4).

- For front side mounting use the bolt (note 7) and washer (note 8) used the original nut, for rear side use bolt (note 5) and washer (note 8).

- Put on the bobbins colour cover, If you have PHV1 or PHV2