T13S

Triumph Street Triple 675 2013-2016

Triumph Street Triple 675R 2013-2016

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Dural. adapter SL d=45mm | 313 | 1 |  |  | 4 | Dural. adapter SL d=45mm | 313 | 1 |  |
| 2 | Bolt M10x75x1,25 | 938 | 1 |  |  | 2 | Bolt M10x75x1,25mm | 938 | 1 |  |
| 3 | Dural. cylinder d=15mm | 8 | 1 |  |  | 3 | Dural. cylinder d=15mm | 8 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |

Installation:

- Installation of the left and right protector make in gripping about the attached photo above.

- Use ID.2 bolt to mount the SL adapter ID.1, under which you dial the dural. roller ID.3

- Then install the M6 ID.5 slider with M4 bolts ID.6 dural. plate.

- At first finish the one side and then mount the second side.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).