T15

Triumph Thruxton 1200/R 2016-

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No. | Pcs |  |  | ID | Description | Cat.No. | Pcs. |  |
| 1 | Aluminium Insert M10 | X1 | 1 |  |  | 1 | Aluminium Insert M10 | X1 | 1 |  |
| 2 | Dural. Nut M22x1,5mm | 446 | 1 |  |  | 2 | Dural. Nut M22x1,5mm | 446 | 1 |  |
| 3 | Dural. Bolt M22x1,5mm | 447 | 1 |  |  | 3 | Dural. Bolt M22x1,5mm | 447 | 1 |  |
| 4 | Bolt M12x150x1,25mm | 984 | 1 |  |  | 4 | Bolt M12x160x1,25mm | 949 | 1 |  |
| 5 | Steel Adapter | 755 | 1 |  |  | 5 | Steel Adapter | 756 | 1 |  |
| 6 | Bolt M6x60mm | 872 | 2 |  |  | 6 | Bolt M6x50mm | 867 | 2 |  |
| 7 | Dural. roller d=86,5mm | 1319 | 1 |  |  | 7 | Dural. Roller d=91,5mm | 1320 | 1 |  |
| 8 | Bolt M10x40mm | 854 | 1 |  |  | 8 | Bolt M10x40mm | 854 | 1 |  |
| 9 | Washer Flat M6 | 947 | 3 |  |  | 9 | Washer Flat M6 | 947 | 3 |  |

9

9

1

1

2

2

3

3

4

4

5

5

6

6

7

7

8

8

Mounting Instructions – Left Side:

- Unscrew the original M12 bolt.

- Unscrew the original M6 bolts from engine side.

- Take the ID.2 dural. nut and ID.4 M12 bolt and push through the bolt through the nut and ID.5 steel adapter. (bigger protector on the left side)

- Mount this assembly into original position of the M12 bolt, but dont tighten it yet.

- Equip the M6 ID.6 bolts with ID.9 flat washers.

- Push these bolts through the steel adapter and tighten. Into the space of upper bolt, between the steel adapter and engine, insert another ID.9 M6 washer.

- Now tighten even the M12 bolt.

- Mount the ID.3 dural. bolt into dural. nut on the steel adapter and use medium strength thread glue. Tighten the nut.

- Now you can proceed with mounting of the protector itself. Insert ID.1 aluminum insert and ID.8 M10 bolt into the protector.

- Full assembly including the protector mount into the M10 thread in the dural. bolt and again, don’t forget to use the thread glue for better performance.

Note: We recommend using medium strenght thread glue during assembly (blue color).

Mounting Instructions – Right Side:

- Unscrew the original M12 bolt.

- Unscrew the original M6 bolts from engine side.

- Take the ID.2 dural. nut and ID.4 M12 bolt and push through the bolt through the nut and ID.5 steel adapter. (smaller protector for right side)

- Mount this assembly into original position of the M12 bolt, but dont tighten it yet.

- Equip the M6 ID.6 bolts with ID.9 flat washers.

- Push these bolts through the steel adapter and tighten.

- Now tighten even the M12 bolt.

- Mount the ID.3 dural. bolt into dural. nut on the steel adapter and use medium strength thread glue. Tighten the nut.

- Now you can proceed with mounting of the protector itself. Insert ID.1 aluminum insert and ID.8 M10 bolt into the protector.

- Full assembly including the protector mount into the M10 thread in the dural. bolt and again, don’t forget to use the thread glue for better performance

Note: We recommend using medium strenght thread glue during assembly (blue color).