T16SD

Triumph Speed Triple 1050 2016-

| Kit for Left Side: |  |  |  |  | Kit for Right Side: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no. | Pcs |  |  | ID. | Description | Cat.no. | Pcs |  |  |
| 1 | Dural. adapter for SLD | 537 | 1 |  |  | 1 | Dural. adapter for SLD | 537 | 1 |  |  |
| 2 | Dural. roller d=50mm | DV13 | 1 |  |  | 2 | Dural. roller d=50mm | DV13 | 1 |  |  |
| 3 | Bolt l M10x70mm | 1007 | 1 |  |  | 3 | Bolt l M10x70mm | 1007 | 1 |  |  |
| 4 | Bolt M8x25 | 815 | 2 |  |  | 4 | Bolt M8x25 | 815 | 2 |  |  |
| 5 | Washer M8 | 802 | 2 |  |  | 5 | Washer M8 | 802 | 2 |  |  |
| 6 | Dural. nut M22x1,5mm | 446 | 1 |  |  | 6 | Dural. nut M22x1,5mm | 446 | 1 |  |  |
| 7 | Dural. bolt M22x1,5mm | 447 | 1 |  |  | 7 | Dural. bolt M22x1,5mm | 447 | 1 |  |  |
| 8 | Bolt M12x80mm | 956 | 1 |  |  | 8 | Bolt M12x80mm | 956 | 1 |  |  |
| 9 | Washer M12 | 803 | 1 |  |  | 9 | Washer M12 | 803 | 1 |  |  |
| 10 | Nut M12 | 811 | 1 |  |  | 10 | Nut M12 | 811 | 1 |  |  |
| 11 | Washer - Spring M10 | 801 | 1 |  |  | 11 | Washer - Spring M10 | 801 | 1 |  |  |

Mounting Instructions – Left Side

- Unscrew the original M12 bolt from motorcycle frame (attached to rear side by nut).

- Use the special ID.6 nut and insert M12 ID.8 bolt into it.

- Take this assembly and mount it onto the motorcycle frame, using ID.9 washer and ID.10 nut on rear side.

- Now screw the special ID.7 bolt into the nut.

- Push the M10x70 ID.3 bolt (with ID.11 washer) through the ID.1 adapter and ID.2 dural roller. Bring the adapter closer into the thread with the dural. bolt.

- Mount and tighten the slider using ID.4 bolts, ID.5 washer.

Use the same procedure for Right Side.

We recommend using medium strenght thread glue during assembly.