T2-2SD

Triumph Speed Triple 1050´05-´10

Fitting kit left side:				          Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Bolt M12x70x1,25 | 937 | 1 |
| 2 | Bracket for SLD | 537 | 1 |
| 3 | Dural. cylinder | 148 | 1 |
| 4 | Dural. reduction | 149 | 1 |
| 5 | Washer M8 | 802 | 2 |
| 6 | Bolt M8 | 815 | 2 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Bolt M12x100 | 986 | 1 |
| 2 | Bracket for SLD | 537 | 1 |
| 3 | Dural. cylinder | 148 | 1 |
| 4 | Dural. reduction | 149 | 1 |
| 5 | Nut M12 | 811 | 1 |
| 6 | Washer M12 | 803 | 1 |
| 7 | Washer M8 | 802 | 2 |
| 8 | Bolt M8 | 815 | 2 |

Instruction to assembly left side:

- Dismantle orig. bolt M12 from frame.

- Throught the bracket for SLD ID.2, dural. adapter ID.4 and dural.cylinder ID.3 insert bolt ID.1.

- This kit gripping on the frame.

- Use bolts ID.6 and washers ID.5 make assembly SLD.

Instruction to assembly right side:

- Dismantle orig. bolt M12 from frame.

- Throught the bracket for SLD ID.2, dural. adapter ID.4 and dural.cylinder ID.3 insert bolt ID.1.

- This kit gripping on the frame.

- Thighten bolt from back side, use nut and washer ID.5 a ID.6.

- Use bolts ID.8 and washers ID.7 make assembly SLD.

All threads, use glue for bolts- medium strength.

2

36

1

1

4

3

2

4

5

6

Place for protectors- ilustration photo.