T3SDM

Triumph Speed Triple T509/955i

2002-2004

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No | Pcs |  |  | ID | Description | Cat.No | Pcs |  |
| 1 | Dural. adapter under slider | 537/B | 1 |  |  | 1 | Dural. adapter under slider | 537/B | 1 |  |
| 2 | Dural. sleeve l=40mm | 12 | 1 |  |  | 2 | Dural. sleeve l=40mm | 12 | 1 |  |
| 3 | Dural. adapter (left) | 619 | 1 |  |  | 3 | Dural. adapter (right) | 620 | 1 |  |
| 4 | Washer M12 | 803 | 1 |  |  | 4 | Washer M12 | 803 | 2 |  |
| 5 | Bolt M12x40x1,25mm | 933 | 1 |  |  | 5 | Bolt M12x65mm | 963 | 1 |  |
| 6 | Bolt M10x65mm | 943 | 1 |  |  | 6 | Bolt M10x65mm | 943 | 1 |  |
| 7 | Stainless steel bolt M6x20mm | 1024 | 2 |  |  | 7 | Stainless steel bolt M6x20mm | 1024 | 2 |  |
| 8 | Stainless steel washer M6 | 1023 | 2 |  |  | 8 | Stainless steel washer M6 | 1023 | 2 |  |
| 9 | Spring washer M10 | 801 | 1 |  |  | 9 | Spring washer M10 | 801 | 1 |  |
|  |  |  |  |  |  | 10 | Lock nuts M12 | 811 | 1 |  |

ID2

ID2

ID1

ID7

Left adapter ID.3

Right adapter ID.3

ID1

ID4

ID4

ID5

ID5

ID6

ID6

ID7

ID8

ID8

ID9

ID9

ID10

Mounting  - left side:

- Remove original bolt M12 from the motorcycle frame.

- Screw the dural. Adapter (ID3) to the motorcycle frame using M12 bolt (ID5). Do not forget washer M12 (ID4).

- Take the dural. Adapter under the slider (plate, ID1) and dural. Sleeve (ID2)

- Mount on the dural. adapter using M10 bolt (ID6) and spring washer M10 (ID9)

- Install the slider on the dural. adapter under the slider using bolts M6 (ID7) and washers (ID8).

Mounting – right side:

- Remove original bolt M12 from the motorcycle frame.

- Attach the dural. Adapter (ID3) to the motorcycle frame using M12 bolt (ID5) and lock nut M12 (ID 11). Do not forget two washers M12 (ID4).

- Take the dural. Adapter under the slider (plate, ID1) and dural. Sleeve (ID2)

- Mount on the dural. adapter using M10 bolt (ID6) and spring washer M10 (ID9)

- Install the slider on the dural. adapter under the slider using bolts M6 (ID7) and washers (ID8).

Warning - We recommend using medium strength thread glue during assembly.

ILUSTRAČNÍ FOTO MÍSTA MONTÁŽE