T3S

Triumph Speed Triple T509/955i

2002-2004

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | pcs |  |
| 1 | Bolt M10x30 | 882 | 1 |  |  | 1 | Bolt M10x30 | 882 | 1 |  |
| 2 | Slider adapter | DV317 | 1 |  |  | 2 | Slider adapter | DV317 | 1 |  |
| 3.1 | Bolt M12x40x1,25mm | 933 | 1 |  |  | 3.2 | Bolt M12x65mm | 963 | 1 |  |
| 4 | Washer M10 | 801 | 1 |  |  | 4 | Washer M10 | 801 | 1 |  |
| 5.1 | Duralový adapter (left) | 619 | 1 |  |  | 5.2 | Duralový adapter (right) | 620 | 1 |  |
| 6 | Washer M12 | 803 | 1 |  |  | 6 | Washer M12 | 803 | 2 |  |
| 7 | Bolt M6x25 | 677 | 3 |  |  | 7 | Bolt M6x25 | 677 | 3 |  |
| 8 | Bolt M4x12 | 676 | 2 |  |  | 8 | Bolt M4x12 | 676 | 2 |  |
|  |  |  |  |  |  | 9 | Nut M12 (safety) | 811 | 1 |  |

ID8

direction

ID7

ID2

ID1

ID5.1

ID5.2

ID9

ID3.1

ID3.1

ID6

Installation for the left side:

- Remove original M12 bolt from the frame of motorcycle.

- Mount dural adapter (ID5.1) on the frame of motorcycle using bolt M12 (ID3.1). Don’t forget to place M10 washer (ID6).

- Now fit slider adapter (ID2) on the dural adapter (ID5.1) using bolt M10 (ID1).

- Fit slider on the slider adapter using three M6 bolts (ID7).

- Fit coloured cover on the slider using two M4 bolts (ID6).

Installation for the right side:

- Remove original M12 bolt from the frame of motorcycle.

- Mount dural adapter (ID5.1) on the frame of motorcycle using bolt M12 (ID3.1) and safety nut M12 (ID9). Don’t forget to place M10 washer (ID6).

- Now fit slider adapter (ID2) on the dural adapter (ID5.1) using bolt M10 (ID1).

- Mount slider on the slider adapter using three M6 bolts (ID7).

- Mount coloured cover on the slider using two M4 bolts (ID6).

We recommend to use the glue (medium strong) on all threads.