T6S

Triumph Sprint 1050 2005-

Left side mounting kit:		     	Right side mounting kit:

| ID | Description | Cat.No. | Pcs |
| --- | --- | --- | --- |
| 1 | Bolt M6x25 | 877 | 3 |
| 2 | Bolt M4x16 | 876 | 2 |
| 3 | Dural.cylinder 5mm | 294 | 1 |
| 4 | Poly. Cylinder 60mm | X14 | 1 |
| 5 | Bolt M12x110x1,25 | 910 | 1 |
| 6 | Bolt M8x30 | 942 | 1 |
| 7 | Bolt M12x40 | 858 | 1 |
| 8 | Dural. adapter | 345 | 1 |
| 9 | Frame adapter | 594 | 1 |
| 10 | 3M sticker | - | 1 |

| ID | Description | Cat.No. | Pcs |
| --- | --- | --- | --- |
| 1 | Bolt M6x25 | 877 | 3 |
| 2 | Bolt M4x16 | 876 | 2 |
| 3 | Dural. cylinder 15mm | 295 | 1 |
| 4 | Poly. Cylinder 50mm | X13 | 1 |
| 5 | Bolt M12x100x1,25 mm | 911 | 1 |
| 6 | Bolt M8x30 | 942 | 1 |
| 7 | Bolt M12x40 | 858 | 1 |
| 8 | Dural. adapter | 345 | 1 |
| 9 | Frame adapter | 593 | 1 |
| 10 | 3M sticker | - | 1 |

10

4

6

6

9

2

2

8

4

9

8

7

7

1

5

1

5

3

3

Instrucitons:

- We recommend to use the glue on all threads.

- Remove original bolt M12 from the frame of motorcycle.

- Mount poly. Cylinde ID on the adapter ID9 using bolt ID6.

- Insert bolt ID7 from back side into the adapter. Using this bolt fit adapter for slider ID8.

- Insert bolt ID5 with cylinder ID3 into the adapter ID9.

- Mount this kit on the frame of motorcycle.

- Fit slider on the adapter using three bolts M6 (ID1).

- Fit coloured cover on the slider using two bolts M4 (ID2).