T6

Triumph Sprint 1050 2005-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M12 | X2 | 1 |
| 2 | Dural.cylinder d=70 | 420 | 1 |
| 3 | Dural.cylinder d=5 | 294 | 1 |
| 4 | Poly.cylinder d=60 | X14 | 1 |
| 5 | Bolt M12x110x1,25 | 910 | 1 |
| 6 | Bolt M8x30 | 942 | 1 |
| 7 | Bolt M12x40 | 858 | 1 |
| 8 | Bolt M12x50 | 869 | 1 |
| 9 | Dural. adapter | 594 | 1 |
| 10 | 3M Tape |  | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M12 | X2 | 1 |
| 2 | Dural. cylinder d=70mm | 420 | 1 |
| 3 | Dural. cylinder d=15mm | 295 | 1 |
| 4 | Poly.cylinder d=50mm | X13 | 1 |
| 5 | Bolt M12x100x1,25mm | 911 | 1 |
| 6 | Bolt M8x30mm | 942 | 1 |
| 7 | Bolt M12x40mm | 858 | 1 |
| 8 | Bolt M12x50mm | 869 | 1 |
| 9 | Dural. adapter | 593 | 1 |
| 10 | 3M tape |  | 1 |

10

4

6

4

6

2

9

7

8

2

1

7

8

1

1

5

3

5

9

3

Instr.to assembly left side:

- Take out original bolt M12 from frame.

- Gripping Poly cylinder ID.4 on the dural.adapter ID.9 use bolt ID.6.

- Insert bolt ID.7 from back side into the adapter and mount dural. cylinder ID.2.

- Use bolt ID.5, put on the bolt dural. cylinder ID.3.

- Mount this kit on the frame, you can use 3M tape.

- Put fairiing on the bike.

- Take bolt ID.8, insert steel box ID.1 into the protector and mount it on the dural. cylinder.

Instr.to assembly right side:

- Take out original bolt M12 from frame.

- Gripping Poly cylinder ID.4 on the dural.adapter ID.9 use bolt ID.6.

- Insert bolt ID.7 from back side into the adapter and mount dural. cylinder ID.2.

- Use bolt ID.5, put on the bolt dural. cylinder ID.3.

- Mount this kit on the frame, you can use 3M tape.

- Put fairiing on the bike.

- Take bolt ID.8, insert steel box ID.1 into the protector and mount it on the dural. cylinder.

Warning: We recommend use the glue of all threads. Medium strong.