T9S

Triumph Tiger 800 2011-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no | Pcs |  |  | No. | Description | Cat.no | Pcs |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |
| 4 | Slider adapter | DV-318 | 1 |  |  | 4 | Slider adapter | DV-318 | 1 |  |
| 6.1 | Bolt M10x30mm | 882 | 1 |  |  | 6.2 | Bolt M10x55mm | 1002 | 1 |  |
| 8 | Bolt M6x25 | 877 | 3 |  |  | 8 | Bolt M6x25 | 877 | 3 |  |
| 9 | Bolt M4x16 | 876 | 2 |  |  | 9 | Bolt M4x16 | 876 | 2 |  |
| 10.1 | Bolt M10x145x1,25mm | 909 | 1 |  |  | 10.2 | Bolt M10x100x1,25mm | 892 | 1 |  |
| 11.1 | Bolt M10x70x1,25mm | 898 | 1 |  |  | 11.2 | Bolt M10x70x1,25mm | 898 | 1 |  |
| 12.1 | Duralumin adapter | 595 | 1 |  |  | 12.2 | Duralumin adapter | 596 | 1 |  |
| 13.1 | Duralumin cylinder d=83mm | 278 | 1 |  |  | 13.2 | Dural.cylinder d=26mm | 280 | 1 |  |
| 14.1 | Duralumin cylinder d=34mm | 279 | 1 |  |  | 14.2 | Dural.cylinder d=33mm | 281 | 1 |  |
| 15 | Washer M10 (safety) | 801 | 1 |  |  | 15 | Washer M10 (safety) | 801 | 1 |  |
| 16 | Plastic cover - black | X4 | 2 |  |  | 16 | Plastic cover - black | X4 | 2 |  |
|  |  |  |  |  |  | 17 | Tape 3M |  | 1 |  |
|  |  |  | ID14.2 |  |  |  |  |  |  |  |

ID10.2

ID13.1

ID10.1

ID4

ID4

ID6.1

ID10

ID12.1

ID12.2

ID6.2

ID10

ID14.1

ID11.1

ID13.2

ID8

ID8

ID11.2

ID9

ID9

Installation – left side:

- Remove original bolts from the frame of motorcycle.

- Fit dural adapter (ID12.1) on the frame of motorcycle using two bolts M10 (ID10.1 + ID11.1). Don’t forget to place two spacers (ID13.1 + ID14.1) under dural adapter.

- Using bolt M10 (ID6.1) fit slider adapter (ID4) on dural adapter (ID12.1). Don’t forget to use safety washer (ID10).

- Fit slider on slider adapter (ID4) using three bolts M6 (ID8).

- Insert coloured cover into slider. Fit coloured cover using two screws M4 (ID9).

Warning! We recommend to use safety glue on threads of all bolts (medium strong – blue colour).

Installation – right side:

- Remove original bolts from the frame of motorcycle.

- Fit dural adapter (ID12.2) on the frame of motorcycle using two bolts M10 (ID10.2 + ID11.2). Don’t forget to place two spacers (ID13.2 + ID14.2) under dural adapter.

- Insert tape 3M between engine and drual adapter (ID12.2).

- Using bolt M10 (ID6.2) fit slider adapter (ID4) on dural adapter (ID12.2). Don’t forget to use safety washer (ID10).

- Fit slider on slider adapter (ID4) using three bolts M6 (ID8).

- Insert coloured cover into slider. Fit coloured cover using two bolts M4 (ID9).

Warning! We recommend to use safety glue on threads of all bolts (medium strong – blue colour).