Y10S

Yamaha FZS1000 Fazer 2001-2005

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | Dural. cylinder 45mm | 314 | 1 |  |  | 1 | Dural. cylinder 45mm | 314 | 1 |  |
| 2 | Bolt M12x30mm | 856 | 1 |  |  | 2 | Bolt M12x30mm | 856 | 1 |  |
| 3 | Bolt M10x130mm | 883 | 1 |  |  | 3 | Bolt M10x130mm | 883 | 1 |  |
| 4 | Bolt M8x130mm | 884 | 1 |  |  | 4 | Bolt M8x130mm | 884 | 1 |  |
| 5 | Washer M10 (plochá) | 804 | 1 |  |  | 5 | Washer M10 (plochá) | 804 | 1 |  |
| 6 | Nut M10 (pojistná) | 810 | 1 |  |  | 6 | Nut M10 (pojistná) | 810 | 1 |  |
| 7 | Washer M8 (plochá) | 802 | 1 |  |  | 7 | Washer M8 (plochá) | 802 | 1 |  |
| 8 | Nut M8 (pojistná) | 809 | 1 |  |  | 8 | Nut M8 (pojistná) | 809 | 1 |  |
| 9 | Dural adapter | 586 | 1 |  |  | 9 | Dural adapter | 586 | 1 |  |
| 10 | Bolt M6x25mm | 877 | 3 |  |  | 10 | Bolt M6x25mm | 877 | 3 |  |
| 11 | Bolt M4x12mm | 876 | 2 |  |  | 11 | Bolt M4x12mm | 876 | 2 |  |

Instructions:

- Remove original bolt M10 from the frame before engine of motorcycle.

- Insert bolt ID3 with washer ID5 and bolt ID4 with washer ID7 into the adapter ID9.

- Mount this kit on the frame and tighten it using nuts ID6 and ID8.

- Put dural. cylinder ID1 on the bolt ID2 and mount this on the adapter.

- Mount slider on the dural. cylinder using three bolts ID10.

- Fit coloured cover using two bolts ID11.

We recommend to use the glue on all threads (medium strong).