Y11SD

Yamaha FJR 1300 2001-2005

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Dural.bracket | 537 | 1 |
| 2 | Dural.spacer for bracket | 13 | 1 |
| 3 | Dural cylinder d=15 | 1148 | 2 |
| 4 | Bolt M12x80x1,25 | 908 | 2 |
| 5 | Duralumin adapter | 670 | 1 |
| 6 | Washer M8 | 800 | 2 |
| 7 | Bolt M8x25 | 815 | 2 |
| 8 | Bolt M10x70 | 851 | 1 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Dural.bracket | 537 | 1 |
| 2 | Dural.spacer for bracket | 13 | 1 |
| 3 | Dural cylinder d=5 | 1149 | 2 |
| 4 | Bolt M12x65x1,25 | 940 | 2 |
| 5 | Duralumin adapter | 671 | 1 |
| 6 | Washer M8 | 800 | 2 |
| 7 | Bolt M8x25 | 815 | 2 |
| 8 | Bolt M10x70 | 851 | 1 |

4

8

1

8

1

2

2

5

3

7

4

3

6

4

5

Instruction to assembly-left side:

- Take down fairing from left and right side.

- Make assembly on the left side, takeoff bolts from frame.

- Insert dural.cylinders d=15mm into the wholes in frame.

- For fixing the dural. adapter use bolts ID.4 M12x80mm- 55Nm.

Instruction to assembly-right side:

- Make assembly on the right side, takeoff bolts from frame.

- Insert dural.cylinders d=5mm into the wholes in frame.

- For fixing the dural. adapter use bolts ID.4 M12x65mm- 55Nm.

- Put fairings back on the bike.

- Place duralspacer ID.2 under the dural. bracket ID.1. This set mounting on the dural. adapter. Take the bolt ID.8.

- Place slider on the dural bracket and tighten it by bolts with washers ID.6, ID.7

Warning: We recommend use the glue of all threads. Medium strong.