Y11

Yamaha FJR 1300 1998-2005

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Bolt M10x40 | 854 | 1 |
| 3 | Dural cylinder d=15 | 1148 | 2 |
| 4 | Bolt M12x80x1,25 | 908 | 2 |
| 5 | Duralumin adapter | 670 | 1 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Bolt M10x40 | 854 | 1 |
| 3 | Dural cylinder d=5 | 1149 | 2 |
| 4 | Bolt M12x65x1,25 | 940 | 2 |
| 5 | Duralumin adapter | 671 | 1 |

4

4

3

5

3

2

1

1

2

4

3

4

3

Instruction to assembly-left side:

- Take down fairing from left and right side.

- Make assembly on the left side, takeoff bolts from frame.

- Insert dural.cylinders d=15mm into the wholes in frame.

- For fixing the dural. adapter use bolts ID.4 M12x80mm- 55Nm.

Instruction to assembly-right side:

- Make assembly on the right side, takeoff bolts from frame.

- Insert dural.cylinders d=5mm into the wholes in frame.

- For fixing the dural. adapter use bolts ID.4 M12x65mm- 55Nm.

- Put fairings back on the bike.

- Insert steel boxes ID.1 into the protectors.

- Take bolts ID.2 and fixing protectors on the dural.adpaters.

Warning: We recommend use the glue of all threads. Medium strong.