Y13S

Yamaha XJR 1300 1998-

Fitting kit:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Stainless steel rod M10x1,25x515x30x30 | - | 1 |
| 2 | Slider holder | 317 | 2 |
| 3 | Alu. spacer | 33 | 2 |
| 4 | Special nut | X11 | 2 |
| 5 | Bolt M6x25 | 877 | 6 |
| 6 | Bolt M4x12 | 876 | 4 |

2

4

4

2

3

1

5

5

3

6

6

Fitting instruction:

- Dismantl original bolt from chassi and engine.

- Place steel rod ID.1. Insert alu. spacer ID.3 from left and right side.

- Place slider holder ID.2 from left and right side.

- Tighten it by nuts ID.4.

- Place slide rand use bolts ID.5. Insert cover and use bolts ID.6

Note: we recommande use glue of threads. Medim strong.