Y14SDM

Yamaha FZ-1 2006-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no | Pcs |  |  | No. | Description | Cat.no | Pcs |  |
| 1 | SLDM adapter | 537/B | 1 |  |  | 1 | SLDM adapter | 537/B | 1 |  |
| 2 | Bolt M10x100x1,25mm | 892 | 1 |  |  | 2 | Bolt M10x100x1,25mm | 892 | 1 |  |
| 3 | Dural. cylinder d=32mm | 55 | 1 |  |  | 3 | Dural. cylinder d=32mm | 55 | 1 |  |
| 4 | Dural. cylinder (adapter) | 10 | 1 |  |  | 4 | Dural. cylinder (adapter) | 10 | 1 |  |
| 5 | Spring washer M10 | 801 | 1 |  |  | 5 | Spring washer M10 | 801 | 1 |  |
| 6 | Bolt M6x20 | 1024 | 2 |  |  | 6 | Bolt M6x20 | 1024 | 2 |  |
| 7 | Washer M6 | 1023 | 2 |  |  | 7 | Washer M6 | 1023 | 2 |  |

INSTRUCTIONS

- We recommend to use medium strong glue on all threads.

- Unbolt original bolt M10 from the frame of motorcycle.

- Put washer ID5, SLD adapter ID1, cylinder ID3 and cylinder adapter ID4 on bolt ID2.

- Mount this kit into the frame of motorcycle.

- Fit slider using two bolts ID6 and washers ID7.