Y15SD

Yamaha R6 2006-

Fitting kit left side:					Fitting kit right side:

| ID | Description | Cat.N | Qt. |
| --- | --- | --- | --- |
| 1 | Steel adapter | 579 | 1 |
| 2 | Alu. spacer d=10mm | 108 | 1 |
| 3 | Bolt M10x60x1,25 | 896 | 1 |
| 4 | Washer spring M10 | 801 | 1 |
| 5 | Alu. spacer d=60mm | 14 | 1 |
| 6 | Alu. bracket for SLD | 537 | 1 |
| 7 | Bolt M10x90 | 880 | 1 |
| 8 | Washer M10 | 804 | 1 |
| 9 | Lock Nut M10 | 810 | 1 |
| 10 | Bolt M8x25 | 815 | 2 |
| 11 | Washer M8 spring | 800 | 2 |
| 12 | Plastic cover | X4 | 1 |
| 13 | 3M Tape | 3M | 1 |

| ID | Description | Cat.N | Qt. |
| --- | --- | --- | --- |
| 1 | Bolt M10x105x1,25 | 893 | 1 |
| 2 | Washer M10 spring | 801 | 1 |
| 3 | Alu. bracket for SLD | 537 | 1 |
| 4 | Alu. cone adapter | 9 | 1 |
| 5 | Alu. spacer d=40mm | 19 | 1 |
| 6 | Bolt M8x25 | 815 | 2 |
| 7 | Washer M8 spring | 800 | 2 |

6

(No.1)

7

5

1

3

6

7

4

8,9

5

(No.2)

4

3

2

1

2

10

11

12

Fitting kit left side:

- Dismantle original bolt M10 from frame.

- Assembly mounting kit for SLD as shown on the picture (no.1).

- Stick 3M tape on the frame, where is steel adapter touching with chassis.

- Place alu. spacer ID.2 and tighten fitting kit on the frame.

- Place slide and tighten it by bolts ID.10 and washers ID.11.

Fitting kit right side:

- Dismantle original bolt M10 from frame.

- Assembly mounting kit for SLD as shown on the picture (no.2).

- Don´t forget place washer ID.2 under the bolt ID.1.

- Mounting this kit on the chassis.

- Place slide and tighten it by bolts ID.6 and washers ID.7.

We recommande use  glue of bolts during assembly. Medium strong.