Y16SDM

Yamaha R1 2007-2008

Fitting kit left side:		     		 Fitting kit right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Diamond adapter | 537/B | 1 |
| 2 | Bolt M10x70x1,25mm | 898 | 1 |
| 3 | Dural. cylinder d=7mm | 137 | 1 |
| 4 | Dural. cylinder d=55mm | 20 | 1 |
| 5 | Bolt M10x95 | 822 | 1 |
| 6 | Washer M10 (spring) | 801 | 2 |
| 7 | Steel adapter | 597 | 1 |
| 8 | Tape 3M | - | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |
| 11 | Dural adapter | DV10 | 1 |

2,6

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Diamond adapter | 537/B | 1 |
| 2 | Bolt M10x60x1,25mm | 896 | 1 |
| 3 | Dural. cylinder d=5mm | 16 | 1 |
| 4 | Dural.cylinder d=55mm | 20 | 1 |
| 5 | Bolt M10x95 | 822 | 1 |
| 6 | Washer M10 (spring) | 801 | 2 |
| 7 | Steel adapter | 598 | 1 |
| 8 | Tape 3M | - | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |
| 11 | Dural adapter | DV10 | 1 |

Picture 1

ID1

ID2+6

ID11

ID5

ID9+10

Picture 2

ID7

ID5+6

ID3

ID3

ID4

Instructions:

- Remove fairing from both sides

- Remove original bolts from the frame of motorcycle.

- Place 3M tape under the adapter (ID7).

- Mount fitting kits according to the picture 1 and 2.

- Put fairing back.

- In the end, mount protectors according to the picure 3.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

Picture 3