Y16S

Yamaha R1 2007-2008

Left side fitting kit:					Right side fitting kit:

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. slider holder | DV-317 | 1 |
| 2 | Bolt M10x75mm | 1007 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |
| 5 | Bolt M10x70x1,25mm | 898 | 1 |
| 6 | Alu. spacer d=7mm | 137 | 1 |
| 7 | Alu. spacer d=55mm | 20 | 1 |
| 8 | Washer M10 (spring) | 801 | 1 |
| 9 | Steel adapter | 597 | 1 |
| 10 | Tape 3M | - | 1 |

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. slider holder | DV-317 | 1 |
| 2 | Bolt M10x75mm | 1007 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |
| 5 | Bolt M10x60x1,25mm | 896 | 1 |
| 6 | Alu. spacer d=5mm | 16 | 1 |
| 7 | Alu. spacer d=55mm | 20 | 1 |
| 8 | Washer M10 (spring) | 801 | 1 |
| 9 | Steel adapter | 598 | 1 |
| 10 | Tape 3M | - | 1 |

1

10

1

7

7

2

2

8

8

9

9

5

3

5

3

4

4

6

6

Left side instruction:

- Dismantle orig. bolt M10 from chassi.

- Stick 3M tape on the chassi, where is steel adapter touching with chassi.

- Insert ID.6 into the chassi. Place steel adapter and gripped it by bolt ID.5 with washer ID.8.

- Place fairing back the bike.

- Place bolt ID.2 into alu. adapter ID.1 and under the adapter ID.1 insert ID.7 on the bolt.This kit assembly on the steel adapter.

- By bolts ID.3 is mounting crash sliders. Place alu.cover and use bolts ID.4.

Assembly place

Assembly- right side:

- Dismantle orig. bolt M10 from chassi.

- Stick 3M tape on the chassi, where is steel adapter touching with chassi.

- Insert ID.6 into the chassi. Place steel adapter and gripped it by bolt ID.5 with washer ID.8.

- Place fairing back the bike.

- Place bolt ID.2 into alu. adapter ID.1 and under the adapter ID.1 insert ID.7 on the bolt.This kit assembly on the steel adapter.

- By bolts ID.3 is mounting crash sliders. Place alu.cover and use bolts ID.4.

Note: During assembly use medium strong glue of all threads.