Y16

Yamaha R1 2007-2008

2,6

Fitting kit left side:		     		 Fitting kit right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x70x1,25mm | 898 | 1 |
| 3 | Dural. cylinder d=7mm | 137 | 1 |
| 4 | Dural. cylinder d=55mm | 20 | 1 |
| 5 | Bolt M10x90mm | 880 | 1 |
| 6 | Washer M10 (spring) | 801 | 2 |
| 7 | Steel adapter | 597 | 1 |
| 8 | Tape 3M |  | 1 |

5

6

8

7

3

4

3

2,6

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x60x1,25mm | 896 | 1 |
| 3 | Dural. cylinder d=5mm | 16 | 1 |
| 4 | Dural.cylinder d=55mm | 20 | 1 |
| 5 | Bolt M10x90mm | 880 | 1 |
| 6 | Washer M10 (spring) | 801 | 2 |
| 7 | Steel adapter | 598 | 1 |
| 8 | Tape 3M |  | 1 |

5

5

4

6

8

7

1

1

2,6

7

3

6

4

2,6

3

2,6

2,6

Instruction to assembly, left side:

- Take off fairing and orig. bolt M10 from frame.

- Stick 3M ID.8 on the frame, under the adapter ID.7.

- Under the adapter ID.7 insert dural.cylinder ID.3 and use bolt ID.2 and washer ID.6 tighten adapter on frame.

- Check all kit and after put fairing on the bike.

- Put steel box ID.1 with bolt. ID.5 and washer ID.6 into the protector. Under the protector put dural.cylinder ID.4.

- This set mount on the steel adapter ID.7.

Instruction to assembly, right side:

- Take off fairing and orig. bolt M10 from frame.

- Stick 3M ID.8 on the frame, under the adapter ID.7.

- Under the adapter ID.7 insert dural.cylinder ID.3 and use bolt ID.2 and washer ID.6 tighten adapter on frame.

- Check all kit and after put fairing on the bike.

- Put steel box ID.1 with bolt. ID.5 and washer ID.6 into the protector. Under the protector put dural.cylinder ID.4.

- This set mount on the steel adapter ID.7.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).