Y19S

Yamaha TDM900 2002-2006

| Kit for the left side |  |  |  |  |  |  |  |  | Kit for the right side |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no. |  | pcs |  |  |  | ID. |  |  | Description |  | Cat.no |  | pcs |  |  |  |  |
| 1 | slider RDmoto | SL |  | 1 |  |  |  | 1 |  |  | slider RDmoto |  | SL |  | 1 |  |  |  |  |
| 2 | Cover | SLcap.. |  | 1 |  |  |  | 2 |  |  | Cover |  | SLcap.. |  | 1 |  |  |  |  |
| 3 | Slider adapter (cylinder) | DV-314 |  | 1 |  |  |  | 3 |  |  | Slider adapter (cylinder) |  | DV-314 |  | 1 |  |  |  |  |
| 4 | Bolt M6x25 | 877 |  | 3 |  |  |  | 4 |  |  | Bolt M6x25 |  | 877 |  | 3 |  |  |  |  |
| 5 | Bolt M4x16 | 876 |  | 2 |  |  |  | 5 |  |  | Bolt M4x16 |  | 876 |  | 2 |  |  |  |  |
| 6 | Bolt M12x75x1,25mm | 914 |  | 1 |  |  |  | 6 |  |  | Bolt M12x75x1,25mm |  | 914 |  | 1 |  |  |  |  |

Instrucions:

- Remove original bolts from the frame of motorcycle.

- Put slider adapter (ID3) on the bolt (ID6).

- Fit this kit on the frame of motorcycle.

- Fit slider using three bolts (ID4).

- Fit coloured cover using two bolts (ID5).