Y24SD

Yamaha FJR1300

2006-2012

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Bolt M12x110x1,25 | 910 | 1 |
| 2 | SLD adapter | 537 | 1 |
| 3 | Dural. cylinder 15mm | 93 | 1 |
| 4 | Cylinder adapter (black) | 171 | 1 |
| 5 | Bolt M8x25 | 815 | 2 |
| 6 | Washer M8 | 800 | 2 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Bolt M12x120x1,25 | 913 | 1 |
| 2 | SLD adapter | 537 | 1 |
| 3 | Dural. cylinder 25mm | 94 | 1 |
| 4 | Cylinder adapter (black) | 171 | 1 |
| 5 | Bolt M8x25 | 815 | 2 |
| 6 | Washer M8 | 800 | 2 |

Instructions:

- Remove original bolt M10 from the frame.

- Prepare bolt ID1 and put SLD adapter ID2, cylinder adapter ID4 and dural. cylinder ID3 on it. On the right side belongs longer cylinder ID3.

- Mount this kit into the frame of motorcycle.

- Fit slider using two bolts ID5 and washers ID6.

We recommend to use the glue on all threads (medium strong).

Ilustation picture, aprox. place for protector gripping.