Y24S

Yamaha FJR1300

2006-2012

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No. | Pcs. |  |  | ID | Description | Cat. No. | Pcs. |  |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider RDmoto | SL | 1 |  |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |  |
| 3 | Slider adapter | DV-314 | 1 |  |  | 3 | Slider adapter | DV-314 | 1 |  |  |
| 4 | Roller d=15mm | 93 | 1 |  |  | 4 | Roller d=15mm | 93 | 1 |  |  |
| 5 | Bolt M6x25 | 877 | 3 |  |  | 5 | Bolt M6x25 | 877 | 3 |  |  |
| 6 | Bolt M4x16 | 876 | 2 |  |  | 6 | Bolt M4x16 | 876 | 2 |  |  |
| 7 | Bolt M12x95x1,25mm | 900 | 1 |  |  | 7 | Bolt M12x85x1,25mm | 899 | 1 |  |  |

Mounting Instructions: Right side

- Unscrew the original bolt.

- Use the ID. 7 Bolt and ID. 4 roller to attach the ID.3 Adapter. To get the slider in the correct angle, the M6 thread needs to be facing upwards.

- Attach the ID. 1 Slider with ID.5 Bolts.

- Finally, insert the ID.2 cover into your slider and fix with ID. 6 Bolts.

Note – We recommend using medium strenght thread glue for all bolts (blue color).

Mounting Instructions: Left Side

- Unscrew the original bolt.

- Use the ID. 7 Bolt and ID. 4 roller to attach the ID.3 Adapter. To get the slider in the correct angle, the M6 thread needs to be facing upwards.

- Attach the ID. 1 Slider with ID.5 Bolts.

- Finally, insert the ID.2 cover into your slider and fix with ID. 6 Bolts.

Note – We recommend using medium strenght thread glue for all bolts (blue color).