Y28SD

Yamaha XJ6 Diversion F 2010-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Drual. Cylinder 30mm | DV11 | 1 |  |  | 1 | Drual. Cylinder 30mm | DV11 | 1 |  |
| 2 | Bolt M10x50x1,25mm | 921 | 1 |  |  | 2 | Bolt M10x50x1,25mm | 921 | 1 |  |
| 3 | Bolt M10x50mm | 849 | 1 |  |  | 3 | Bolt M10x50mm | 849 | 1 |  |
| 4 | Duralumin adapter | 615 | 1 |  |  | 4 | Duralumin adapter | 616 | 1 |  |
| 5 | Tape 3M | - | - |  |  | 5 | Tape 3M | - | - |  |
| 6 | Plastic cap | X4 | 1 |  |  | 6 | Plastic cap | X4 | 1 |  |
| 7 | Washer M10 (spring) | 801 | 1 |  |  | 7 | Washer M10 (spring) | 801 | 1 |  |
| 8 | SLD adapter | 537 | 1 |  |  | 8 | SLD adapter | 537 | 1 |  |
| 9 | Bolt M8x25mm | 815 | 2 |  |  | 9 | Bolt M8x25mm | 815 | 2 |  |
| 10 | Washer M8 | 800 | 2 |  |  | 10 | Washer M8 | 800 | 2 |  |

ID1

ID8

ID4

ID2

ID3

Instructions

- We recommend to use medium strong glue on all threads.

- Dismantle side fairing and original bolt from the frame according to the picture below.

- Stick 3M tape on the frame of motorcycle to prevent its damage.

- Fit adapter ID4 on the frame using bolt ID2.

- Prepare SLD adapter ID8 and dural. cylinder ID1 and fit it on the steel adapter using bolt M10 (ID3) and washer ID7.

- Fit slider using two bolts ID9 and washers ID10.

- In the end place plastic caps on both bolts.

ILLUSTRATIVE PHOTOS OF Place of the installation