Y31SDM

Yamaha MT-09 ´14

Fittng kit left side:		     		  Fitting kit right side:

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. bracket | 537/B | 1 |
| 2 | Dural. adapter | 12 | 1 |
| 3 | Aluminium spacer | 1127 | 1 |
| 4 | Bolt M10x110x1,25 | 928 | 1 |
| 5 | Washer M6 | 1023 | 2 |
| 6 | Bolt M6x20 | 1024 | 2 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Dural. bracket | 537/B | 1 |
| 2 | Dural. adapter | 10 | 1 |
| 3 | Aluminium spacer | 1127 | 1 |
| 4 | Bolt M10x85x1,25 | 934 | 1 |
| 5 | Washer M6 | 800 | 2 |
| 6 | Bolt M6x20 | 815 | 2 |

Instruction to assembly:

- Take out original bolt from frame.

- Insert bolt ID.4 through the dural. bracket ID.1 and dural. adapter ID.2. Into the frame insert alu spacer ID.3

- This kit assembly on the frame.

- Place slide and tighten it by bolts ID.6 and washers ID.5.

Instruction to assembly right side:

- Take out original bolt from frame.

- Insert bolt ID.4 through the dural. bracket ID.1 and dural. adapter ID.2. Into the frame insert alu spacer ID.3

- This kit assembly on the frame.

- Place slide and tighten it by bolts ID.6 and washers ID.5.

1

1

2

3

2

3

4

4

1

5

1

6

Warning: Use the glue of all threads. Medium strong.