Y31S

Yamaha MT-09 ´14-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 1 | Bolt M6x25 | 877 | 3 |
| 2 | Bolt M4x16 | 876 | 2 |
| 3 | Dural.spacer d=25 | 1126 | 1 |
| 4 | Bolt M10x90x1,25 | 890 | 1 |
| 5 | Dural. adapter | 292 | 1 |

| ID | Description | Cat.n | Qt |
| --- | --- | --- | --- |
| 1 | Bolt M6x25 | 877 | 3 |
| 2 | Bolt M4x16 | 876 | 2 |
| 3 | Dural.spacer d=13 | 1127 | 1 |
| 4 | Bolt M10x80x1,25 | 920 | 1 |
| 5 | Dural. adapter | 292 | 1 |

Instruction to assembly:

- Take out off original bolt from frame.

- Left hand side:

- Bolt ID.4 insert trough the dural.adapter ID.5.

- Place dural spacer ID.3 on the bolt.

- Place spacer ID.3 into the frame.

- Assembly bolt with kit of adapters into the frame.

- Take bolts ID.1 and mounting slider. Into the protector insert alu. cap and tighten it by bolts ID.2.

- Take out off original bolt from frame.

- Right hand side:

- Bolt ID.4 insert trough the dural.adapter ID.5.

- Place spacer ID.3 into the frame.

- Assembly bolt with kit of adapters into the frame.

- Take bolts ID.1 and mounting slider. Into the protector insert alu. cap and tighten it by bolts ID.2.

Use medium glue of all threads.

4

2

2

1

1

4

3

5

5

3