Y33SDM

Yamaha MT07 2014

Kit for leesft side:				    Kit for right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 2 | Dural. Cylinder | 1115 | 1 |
| 3 | Bracket for SLD | 537 | 1 |
| 4 | Bolt M6x20 | 1024 | 2 |
| 5 | Washer M6 | 1023 | 2 |
| 6 | Bolt M12x110x1,25 | 910 | 1 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 2 | Dural. Cylinder | 1115 | 1 |
| 3 | Bracket for SLD | 537 | 1 |
| 4 | Bolt M6x20 | 1024 | 2 |
| 5 | Washer M6 | 1023 | 2 |
| 6 | Bolt M12x110x1,25 | 910 | 1 |

Instruction to assemble left side:

- Dismantle original bolt from frame.

- Insert duralumin cylinder ID2 into the frame.

- On the dural. Cylinder assemble bracket for SLD (ID3) use the bolt (ID6).

- Mounting slider (ID1) use the bolt (ID4) amd washer (ID5).

Instruction to assemble right side:

- Dismantle original bolt from frame.

- Insert duralumin cylinder ID2 into the frame.

- On the dural. Cylinder assemble bracket for SLD (ID3) use the bolt (ID6).

- Mounting slider (ID1) use the bolt (ID4) amd washer (ID5).

3

2

4,5

6

Use the glue on the all threads.