Y34SDM

Yamaha R3/250 ´14-

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x90x1,25 | 890 | 1 |
| 2 | Bolt M10x85x1,25 | 934 | 1 |
| 3 | Dural. spacer 21mm | 1163 | 1 |
| 4 | Dural. spacer 30mm | DV3 | 1 |
| 5 | Dural. bracket | 686 | 1 |
| 6 | Dural. adapter 60mm | DV14 | 1 |
| 7 | Dural holder pro SLDM | 537/B | 1 |
| 8 | Bolt M10x85 | 981 | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x90x1,25 | 890 | 1 |
| 2 | Bolt M10x85x1,25 | 934 | 1 |
| 3 | Dural. spacer 21mm | 1163 | 1 |
| 4 | Dural. spacer 30mm | DV3 | 1 |
| 5 | Dural. bracket | 687 | 1 |
| 6 | Dural. adapter 60mm | DV14 | 1 |
| 7 | Dural holder pro SLDM | 537/B | 1 |
| 8 | Bolt M10x85 | 981 | 1 |
| 9 | Bolt M6x20 | 1024 | 2 |
| 10 | Washer M6 | 1023 | 2 |

1

1

3

3

5

5

7

7

1

9,10

6

6

2

4

2

8

8

4

9,10

Instructions:

- Remove fairing from the left side. Remove left side original bolts from the frame according to the picture.

- Prepare dural spacer ID3 and ID4. Insert these dural spacers between frame and dural bracket ID5. Tighten dural bracket using screws ID1 and ID2 according to the picture.

- Put fairing back.

- Remove fairing from the right side. Remove right side original bolts from the frame according to the picture.

- Prepare dural spacer ID3 and ID4. Insert these dural spacers between frame and dural bracket ID5. Tighten dural bracket using screws ID1 and ID2 according to the picture.

- Thighten both bolts and put fairing back of the bike.

- For both sides:

  - Fit dural adapter ID6 and dural holder for SLD ID7 on the bracket ID7 using one bolt ID8.

  - In the end fit slider using bolts ID9 and washers ID10.

Warning: We recommend use the glue of all threads. Medium strong.