Y34S

Yamaha R3/250 ´14-

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x90x1,25 | 890 | 1 |
| 2 | Bolt M10x85x1,25 | 934 | 1 |
| 3 | Dural. spacer 21mm | 1163 | 1 |
| 4 | Dural. spacer 30mm | DV3 | 1 |
| 5 | Dural. bracket | 686 | 1 |
| 6 | Dural. adapter | 318 | 1 |
| 7 | Bolt M10x30 | 882 | 1 |
| 8 | Bolt M6x25 | 877 | 3 |
| 9 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x90x1,25 | 890 | 1 |
| 2 | Bolt M10x85x1,25 | 934 | 1 |
| 3 | Dural. spacer 21mm | 1163 | 1 |
| 4 | Dural. spacer 30mm | DV3 | 1 |
| 5 | Dural. bracket | 687 | 1 |
| 6 | Dural. adapter | 318 | 1 |
| 7 | Bolt M10x30 | 882 | 1 |
| 8 | Bolt M6x25 | 877 | 3 |
| 9 | Bolt M4x16 | 876 | 2 |

3

3

1

1

5

7

7

5

6

6

4

2

8

8

9

2

4

9

Instruction to assembly:

- Remove left side fairing of the bike. Remove left side original bolts of the frame as shown arraws on the picture.

- Put dural. spacer ID.3 into the frame (front) and sligtly tighten bolt ID.1. Between the dural. bracket ID.5 and engine bracket insert spacer ID.4. Use bolt ID.2.

- Thighten both bolts and put fairing back of the bike.

- Remove right side fairing of the bike. Remove right side original bolts of the frame as shown arraws on the picture.

- Put dural. spacer ID.3 into the frame (front) and sligtly tighten bolt ID.1. Between the dural. bracket ID.5 and engine bracket insert spacer ID.4. Use bolt ID.2.

- Thighten both bolts and put fairing back of the bike.

- Left side: For mounting dural. adapter ID.6 use bolt ID.7. Put slider on the adapter and tighten it by bolts ID.8. Insert aluminum cover and tighten it by bolts ID.9. Right side: For mounting dural. adapter ID.6 use bolt ID.7. Put slider on the adapter and tighten it by bolts ID.8. Insert aluminum cover and tighten it by bolts ID.9.

Warning: We recommend use the glue of all threads. Medium strong.