Y35

Yamaha MT-09 Tracer ´14

Fittng kit left side:		     		  Fitting kit right side:

| ID | Description | Cat. | QTY |
| --- | --- | --- | --- |
| 2 | Steel box | X1 | 1 |
| 3 | Al.spacer d=25mm | 1126 | 1 |
| 4 | Bolt M10x95x1,25 | 927 | 1 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 2 | Steel box | X1 | 1 |
| 3 | Alu.spacer d=13mm | 1127 | 1 |
| 4 | Bolt M10x85x1,25 | 934 | 1 |

Instruction to assembly left side:

- Take out original bolt from frame.

- Insert steel box ID.2 into the protektor ID.1.

- Put bolt ID.4 into the protector, unrer the protector put dural.spacer ID.5 and into the frame insert ID.3.

- This set mount into the frame.

Instruction to assembly right side:

- Take out original bolt from frame.

- Insert steel box ID.2 into the protektor ID.1.

- Put bolt ID4 into the protector and put on it al.cylinder ID.3.

- This set mount into the frame.

Warning: Use the glue of all threads. Medium strong.