Y36SD

Yamaha R1 2015

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Dural.bracket for SLD | 537 | 1 |
| 2 | Bolt M12x110x1,25 | 910 | 1 |
| 3 | Alu.spacer d=60mm | 152 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Dural.bracket for SLD | 537 | 1 |
| 2 | Bolt M12x90x1,25 | 931 | 1 |
| 3 | Alu. spacer d=35mm | 153 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Dural. roller | 1260 | 1 |

Instruction to assembly:

- The first do assembly left side.

- Please leave all spacer´s/bushes in place.

- Take the dural. bracket ID.1 for left side, insert bolt ID.2 through the bracket and aluminum spacer ID.3.

- This kit gripping on the frame.

- Place slider and for tighten take bolts ID.4 and washers ID.5

- Mounting instruction for right side.

- Please leave all spacer´s/bushes in place.

- Take the dural. bracket ID.1, insert bolt ID.2 through the bracket and aluminum spacer ID.3.

- Gripping it on the frame.

- Place slider and for tighten take bolts ID.4 and washers ID.5

- For the right position and assembly check the pictures.

Warning: We recommend use the glue of all threads. Medium strong.

1

3

2

1

2

3

4

4

5

5