Y36S

Yamaha R1 2015

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Alu. spacer for SL | 348 | 1 |
| 2 | Bolt 12x95x1,25 | 900 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Alu. spacer for SL | 1137 | 1 |
| 2 | Bolt M12x60x1,25 | 916 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |
| 5 | Alu. Roller d=5mm | 1260 | 1 |

Instruction to assembly:

- The first do assembly left side.

- Take off Orignal bolt.

- Leave all spacer´s/bushes in place.

- Take the alu. spacer ID.1 for left side, insert bolt ID.2 into the spacer ID.1.

- This kit gripping on the frame and tighten.

- Place slide and tighten it by bolts ID.3, place alu. cover into the slider use bolts ID.4.

- Mounting instruction for right side.

- Take off Orignal bolt.

- Please leave all spacer´s/bushes in place.

- Take the alu. spacer ID.1 for right side insert bolt ID.2 into this kit.

- This kit gripping on the frame and tighten.

- Place slide and tighten it by bolts ID.3, place alu. cover into the slider use bolts ID.4.

Warning: We recommend use the glue of all threads. Medium strong.

1

1

3

4

4

3

5

2