Y37SDM

Yamaha FZ-1

Kit for left side:		     		            Kit for right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. holder | 697 | 1 |
| 2 | Spacer d=20 | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cap | X4 | 1 |
| 6 | 3M Tape | 3M | 1 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Alu. spacer for slider | 9 | 1 |
| 9 | Alu. holder for SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. holder | 696 | 1 |
| 2 | Spacer d=20 | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cap | X4 | 1 |
| 6 | 3M Tape | 3M | 1 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Alu. spacer for slider | 9 | 1 |
| 9 | Alu. holder for SLDM | 537/B | 1 |
| 10 | Bolt M6x20 | 1024 | 2 |
| 11 | Washer M6 | 1023 | 2 |

Assembly instruction:

- Dismantle orig. bolts M10 from chassi.

- Place alu. spacers ID.2 into the chassi hole.

- Alu. adapters ID. grip on the chassi by bolts and washers ID.3, ID.4.

- You can stick 3M tape on the chassi in touching place.

- Stainless pin has to be fixed, it´s save protector station during crash.

- Bolt ID.7 insert throught the alu. holder ID.9 and spacer ID.8. This kit assembly.

- Place slider and fix it by bolts ID.10 and washer ID.11.

Done left side first.

Use medium strong glue for threads during assembly.

1

8

6

1

8

2

4

3

10

9

7

4

2

7

3

9

10

11

11