Y37S

Yamaha FZ-1

| Kit for the left side | Kit for the right side |
| --- | --- |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 697 | 1 |
| 2 | Dural. cylinder 20 mm | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cover | X4 | 1 |
| 6 | 3M tape | 3M | 1 |
| 7 | SL01 adapter | 317 | 1 |
| 8 | Bolt M10x25 | 868 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter | 696 | 1 |
| 2 | Dural. cylinder 20 mm | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cover | X4 | 1 |
| 6 | 3M tape | 3M | 1 |
| 7 | SL01 adapter | 317 | 1 |
| 8 | Bolt M10x25 | 868 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

Instructions:

- Remove original bolt M10 from the frame of motorcycle.

- Stick 3M tape on the frame.

- Insert cylinder ID2 into the frame.

- Insert bolt ID3 with washer ID4 into the adapter ID1.

- Mount this kit into the frame.

- Insert bolt ID8 into the dural adapter ID7 and mount this kit on the adapter ID1.

- Fit slider using three bolts ID9.

- Fit coloured cover using two bolts ID10.

1

8

6

1

8

2

4

3

10

9

7

4

2

7

3

9

10

We recommend to use the glue on all threads (medium strong).