Y37

Yamaha FZ-1

Kit for the left side:		     		            Kit for the right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. bracket | 697 | 1 |
| 2 | Alu. spacer d=20 | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cap | X4 | 1 |
| 6 | 3M tape | 3M | 2 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Steel box | X1 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Alu. bracket | 696 | 1 |
| 2 | Alu. spacer d=20 | 435 | 1 |
| 3 | Bolt M10x70x1,25 | 898 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Plastic cap | X4 | 1 |
| 6 | 3M tape | 3M | 2 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Steel box | X1 | 1 |

Assembly instruction:

- Dismantle original bolt M10 from chassi. Make assembly for the left side firstly.

- Place alu. spacer ID.2 into the chassi. Wher is the alu holder touching together chassi, stick 3M tape.

- Place bolt ID.3 and washer ID.4 into the adapter ID.1.

- Wher is a steel pin touching with chassi, stick 3M tape.

- Steel pin rest on chassi and tighten bolt.

- Insert steel box into the protector ID.8 with bolt ID.7.

- Make protectors assembly.

Same assembly make on the oposite site.

Note: Use thread glue on the all bolts. Medium strong.

1

8

6

1

8

2

4

3

7

4

2

7

3